ESP32-S3 Zero Colour Block Detection (Red) Test Results

Platform: Waveshare ESP32-S3 Zero + OV2640 Camera (160×120 RGB565)

Algorithm: Pixel-wise RGB565 colour segmentation | Inference resolution: 160×120

The Waveshare ESP32-S3 Zero was configured with an OV2640 camera module using the ESP32-S3’s built-in DVP camera peripheral and the esp_camera library [2]. Colour block detection was implemented via a pixel-wise RGB565 segmentation algorithm, scanning every second pixel (stride=2) across the 160×120 framebuffer. A bounding box is reported when the number of qualifying pixels meets a minimum hit threshold and the resulting box dimensions are at least 15×15 pixels. Three independent runs were conducted under the fixed camera rig used across all platforms.

Run Conditions (Red Target)

- Run 1 (Static): Red card present at fixed distance with minimal movement.

- Run 2 (Negative control): Red card removed completely from the scene.

- Run 3 (Dynamic): Card moved left/right/up/down for 30 s, then towards and away from the camera for 30 s.

Table 1. Per-Run Benchmark Summary — Colour Block Detection (Red)

| Run | Condition | Samples (N) | Duration (s) | FPS | Valid Detections | Detection Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Static | 770 | 64.052 | 12.021 | 770 | 100.000 |
| 2 | Negative Control | 771 | 63.870 | 12.071 | 0 | 0.000 (FPR) |
| 3 | Dynamic | 822 | 71.018 | 11.575 | 769 | 93.552 |
| Combined | All Runs | 2,363 | 198.940 | 11.878 | 1,539 | — |

Table 2. Bounding Box Statistics (Valid Detections Only) — Red

| Metric (pixels) | Run 1 — Static (Mean ± SD, Min–Max) | Run 3 — Dynamic (Mean ± SD, Min–Max) | Overall (Mean ± SD) |
| --- | --- | --- | --- |
| x (centre) | 72.643 ± 6.999 (57–87) | 77.646 ± 16.440 (33–136) | 75.143 ± 12.873 |
| y (centre) | 66.497 ± 11.433 (56–105) | 71.346 ± 16.204 (37–102) | 68.920 ± 14.225 |
| w (width) | 110.512 ± 15.050 (74–136) | 110.481 ± 22.913 (28–158) | 110.496 ± 19.375 |
| h (height) | 100.101 ± 23.014 (22–118) | 86.182 ± 32.925 (16–118) | 93.146 ± 29.234 |

Combined Summary

- Total samples: 2,363 over 198.940 s. Weighted average FPS: 11.878 FPS.

- Run 1 (Static): 100.000% detection rate — all 770 frames produced a valid bounding box.

- Run 2 (Negative control): 0.000% false positive rate. ✔

- Run 3 (Dynamic): 93.552% detection rate. 53 frames produced no detection during fast motion or extreme distances.

- Bounding box width was highly consistent across static and dynamic conditions (mean ≈ 110 pixels).

Discussion

The ESP32-S3 Zero achieved a consistent frame rate of approximately 12 FPS, a significant improvement over the XIAO ESP32-S3 Sense (~5.5 FPS), attributable to the simpler pixel-wise segmentation algorithm compared to FOMO-based inference. The 100% static detection rate and 0% false positive rate demonstrate robust colour discrimination under controlled conditions. The 6.4% missed frames in Run 3 are consistent with motion blur and partial occlusion reducing qualifying pixels below the minimum hit threshold. The large bounding box dimensions (mean width ≈ 110 pixels on a 160-pixel-wide frame) reflect the deliberate placement of the red target at close range, consistent with the fixed-rig methodology applied across all platforms.

Setup Difficulties and Adjustments

- The originally procured Arducam HM01B0 camera (SKU B0315) was confirmed architecturally incompatible with the ESP32-S3 Zero: its 1-bit serial interface requires PIO-based bit capture hardware available on the RP2040 but absent on the ESP32-S3. A Waveshare OV2640 camera board was procured as a replacement [1].

- Initial framebuffer allocation on QVGA (320×240) RGB565 failed with a malloc error; frame size was reduced to QQVGA (160×120) and the framebuffer located in PSRAM to resolve this.

- The Arduino IDE partition scheme was changed from Default 4MB with SPIFFS to Huge APP (3MB No OTA/1MB SPIFFS) to accommodate the esp_camera library footprint.

- RGB565 byte-swap was enabled in the detection algorithm to correct for the OV2640’s byte order on the ESP32-S3 DVP interface [2].

Conclusion

The ESP32-S3 Zero successfully performed red colour block detection at 11.878 FPS weighted average, achieving 100% static detection rate and 0% false positive rate. These results confirm the viability of the ESP32-S3 Zero for real-time colour-based detection tasks using the OV2640 camera and pixel-wise RGB565 segmentation.

#include "esp_camera.h"

// Settings for colour detection

#define TARGET_COLOUR   1         // 1=RED, 2=GREEN, 3=BLUE

#define RGB565_SWAP     1         // OV2640 on ESP32-S3 Zero - swap enabled

#define RUN_NUMBER      1

static const uint32_t WARMUP_MS = 5000;

// === ESP32-S3 Zero + OV2640 pins ===

#define PWDN_GPIO_NUM    16

#define RESET_GPIO_NUM   15

#define XCLK_GPIO_NUM     6

#define SIOD_GPIO_NUM     1

#define SIOC_GPIO_NUM     2

#define Y9_GPIO_NUM      14

#define Y8_GPIO_NUM      13

#define Y7_GPIO_NUM      12

#define Y6_GPIO_NUM      11

#define Y5_GPIO_NUM      10

#define Y4_GPIO_NUM       9

#define Y3_GPIO_NUM       8

#define Y2_GPIO_NUM       7

#define VSYNC_GPIO_NUM    3

#define HREF_GPIO_NUM     4

#define PCLK_GPIO_NUM     5

static uint32_t t0 = 0;

static inline uint16_t px16(const uint16_t *buf, int idx) {

uint16_t p = buf[idx];

#if RGB565_SWAP

p = (p >> 8) | (p << 8);

#endif

return p;

}

static inline bool isTarget(uint16_t p) {

uint8_t R = (p >> 11) & 0x1F; R = (R << 3) | (R >> 2);

uint8_t G = (p >>  5) & 0x3F; G = (G << 2) | (G >> 4);

uint8_t B = (p      ) & 0x1F; B = (B << 3) | (B >> 2);

#if TARGET_COLOUR == 1  // RED

return (R > 100) && (R > G + 35) && (R > B + 35);

#elif TARGET_COLOUR == 2  // GREEN

return (G > 100) && (G > R + 35) && (G > B + 35);

#elif TARGET_COLOUR == 3  // BLUE

return (B > 100) && (B > R + 35) && (B > G + 35);

#endif

}

void setup() {

Serial.begin(115200);

delay(500);

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer   = LEDC_TIMER_0;

config.pin_d0       = Y2_GPIO_NUM;

config.pin_d1       = Y3_GPIO_NUM;

config.pin_d2       = Y4_GPIO_NUM;

config.pin_d3       = Y5_GPIO_NUM;

config.pin_d4       = Y6_GPIO_NUM;

config.pin_d5       = Y7_GPIO_NUM;

config.pin_d6       = Y8_GPIO_NUM;

config.pin_d7       = Y9_GPIO_NUM;

config.pin_xclk     = XCLK_GPIO_NUM;

config.pin_pclk     = PCLK_GPIO_NUM;

config.pin_vsync    = VSYNC_GPIO_NUM;

config.pin_href     = HREF_GPIO_NUM;

config.pin_sccb_sda = SIOD_GPIO_NUM;

config.pin_sccb_scl = SIOC_GPIO_NUM;

config.pin_pwdn     = PWDN_GPIO_NUM;

config.pin_reset    = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_RGB565;

config.frame_size   = FRAMESIZE_QQVGA;  // 160x120

config.fb_count     = 1;

config.fb_location  = CAMERA_FB_IN_PSRAM;

config.grab_mode    = CAMERA_GRAB_LATEST;

if (esp_camera_init(&config) != ESP_OK) {

while (1) { Serial.println("Camera Init Failed"); delay(1000); }

}

sensor_t *s = esp_camera_sensor_get();

s->set_whitebal(s, 1);

s->set_saturation(s, 2);

s->set_brightness(s, 0);

s->set_contrast(s, 1);

s->set_vflip(s, 0);

s->set_hmirror(s, 0);

Serial.println("Warming up sensor...");

delay(WARMUP_MS);

s->set_whitebal(s, 0);  // lock white balance after warmup

t0 = millis();

Serial.println("time_ms,det_count,colour_id,x,y,w,h");

}

void loop() {

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) return;

uint32_t time_ms = millis() - t0;

const int W = fb->width;

const int H = fb->height;

const uint16_t *buf = (const uint16_t*)fb->buf;

int xmin = W, ymin = H, xmax = -1, ymax = -1;

int hits = 0;

const int stride = 2;

const int MIN_HITS = 20;  // lowered for 160x120 resolution

for (int y = 0; y < H; y += stride) {

int row = y * W;

for (int x = 0; x < W; x += stride) {

uint16_t p = px16(buf, row + x);

if (isTarget(p)) {

hits++;

if (x < xmin) xmin = x;

if (y < ymin) ymin = y;

if (x > xmax) xmax = x;

if (y > ymax) ymax = y;

}

}

}

int det_count = 0;

int cx = -1, cy = -1, bw = -1, bh = -1;

if (hits >= MIN_HITS) {

bw = xmax - xmin;

bh = ymax - ymin;

if (bw >= 15 && bh >= 15) {

det_count = 1;

cx = xmin + bw / 2;

cy = ymin + bh / 2;

} else {

bw = bh = cx = cy = -1;

}

}

Serial.printf("%lu,%d,%d,%d,%d,%d,%d\n",

time_ms, det_count, TARGET_COLOUR, cx, cy, bw, bh);

esp_camera_fb_return(fb);

delay(10);

}

ESP32-S3 Zero Colour Block Detection (Green) Test Results

Platform: Waveshare ESP32-S3 Zero + OV2640 Camera (160×120 RGB565)

Algorithm: Pixel-wise RGB565 colour segmentation | Thresholds tuned for medium-dark green (G > 80, G > R + 25, G > B + 15)

The Waveshare ESP32-S3 Zero was configured with an OV2640 camera module using the ESP32-S3’s built-in DVP camera peripheral and the esp_camera library. Green colour block detection was implemented using a pixel-wise RGB565 segmentation algorithm with thresholds tuned for a medium-dark green target. Three independent runs were conducted under the same fixed camera rig used across all platforms.

Run Conditions (Green Target)

- Run 1 (Static): Green card present at fixed distance with minimal movement.

- Run 2 (Negative control): Green card removed completely from the scene.

- Run 3 (Dynamic): Card moved left/right/up/down for 30 s, then towards and away from the camera for 30 s.

Table 1. Per-Run Benchmark Summary — Colour Block Detection (Green)

| Run | Condition | Samples (N) | Duration (s) | FPS | Valid Detections | Detection Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Static | 656 | 125.544 | 5.225 | 656 | 100.000 |
| 2 | Negative Control | 681 | 69.609 | 9.783 | 0 | 0.000 (FPR) |
| 3 | Dynamic | 657 | 66.719 | 9.847 | 651 | 99.087 |
| Combined | All Runs | 1,994 | 261.872 | 7.615 | 1,307 | — |

Table 2. Bounding Box Statistics (Valid Detections Only) — Green

| Metric (pixels) | Run 1 — Static (Mean ± SD, Min–Max) | Run 3 — Dynamic (Mean ± SD, Min–Max) | Overall (Mean ± SD) |
| --- | --- | --- | --- |
| x (centre) | 87.960 ± 2.987 (81–92) | 72.419 ± 12.852 (28–98) | 74.545 ± 15.082 |
| y (centre) | 50.684 ± 5.077 (33–57) | 50.842 ± 9.471 (13–63) | 51.841 ± 7.392 |
| w (width) | 124.098 ± 5.949 (116–138) | 137.257 ± 23.466 (56–158) | 119.186 ± 26.932 |
| h (height) | 101.369 ± 10.155 (66–114) | 100.885 ± 19.160 (26–118) | 94.690 ± 18.452 |

Combined Summary

- Total samples: 1,994 over 261.872 s. Weighted average FPS: 7.615 FPS.

- Run 1 (Static): 100.000% detection rate — all 656 frames produced a valid bounding box.

- Run 2 (Negative control): 0.000% false positive rate — 0 of 681 frames triggered spurious detections. ✔

- Run 3 (Dynamic): 99.087% detection rate across 657 frames; 6 frames produced no detection.

- Run 1 exhibited notably lower FPS (5.225) compared to Runs 2 and 3 (~9.85 FPS), due to the green card filling the majority of the 160×120 frame causing the per-pixel hit evaluation loop to process a larger proportion of qualifying pixels per frame.

Discussion

The green detection results reveal a fundamental challenge with pixel-wise RGB565 segmentation applied to medium-dark, less-saturated colours. The green target required relaxed thresholds (G > 80 vs R > 100 for red; margin reduced from +50 to +25/+15) to achieve reliable detection. Despite these relaxed thresholds, the negative control produced a 0.000% false positive rate, confirming effective discrimination in this test environment. The static and dynamic detection rates (100% and 99.087%) confirm the thresholds are sufficient to detect the target when present. The trade-off between sensitivity and specificity is a documented characteristic of threshold-based colour detectors on hardware-constrained embedded platforms [3].

Conclusion

The ESP32-S3 Zero achieved a weighted average of 7.615 FPS across three green colour detection benchmark runs, with 100% static and 99.087% dynamic detection rates and a 0.000% false positive rate.

#include "esp_camera.h"

#define TARGET_COLOUR   2         // 2=GREEN

#define RGB565_SWAP     1

#define RUN_NUMBER      1

static const uint32_t WARMUP_MS = 5000;

#define PWDN_GPIO_NUM    16

#define RESET_GPIO_NUM   15

#define XCLK_GPIO_NUM     6

#define SIOD_GPIO_NUM     1

#define SIOC_GPIO_NUM     2

#define Y9_GPIO_NUM      14

#define Y8_GPIO_NUM      13

#define Y7_GPIO_NUM      12

#define Y6_GPIO_NUM      11

#define Y5_GPIO_NUM      10

#define Y4_GPIO_NUM       9

#define Y3_GPIO_NUM       8

#define Y2_GPIO_NUM       7

#define VSYNC_GPIO_NUM    3

#define HREF_GPIO_NUM     4

#define PCLK_GPIO_NUM     5

static uint32_t t0 = 0;

static inline uint16_t px16(const uint16_t *buf, int idx) {

uint16_t p = buf[idx];

#if RGB565_SWAP

p = (p >> 8) | (p << 8);

#endif

return p;

}

static inline bool isTarget(uint16_t p) {

uint8_t R = (p >> 11) & 0x1F; R = (R << 3) | (R >> 2);

uint8_t G = (p >>  5) & 0x3F; G = (G << 2) | (G >> 4);

uint8_t B = (p      ) & 0x1F; B = (B << 3) | (B >> 2);

#if TARGET_COLOUR == 2  // GREEN - tuned for medium-dark green

return (G > 80) && (G > R + 25) && (G > B + 15);

#endif

}

void setup() {

Serial.begin(115200);

delay(500);

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer   = LEDC_TIMER_0;

config.pin_d0       = Y2_GPIO_NUM;

config.pin_d1       = Y3_GPIO_NUM;

config.pin_d2       = Y4_GPIO_NUM;

config.pin_d3       = Y5_GPIO_NUM;

config.pin_d4       = Y6_GPIO_NUM;

config.pin_d5       = Y7_GPIO_NUM;

config.pin_d6       = Y8_GPIO_NUM;

config.pin_d7       = Y9_GPIO_NUM;

config.pin_xclk     = XCLK_GPIO_NUM;

config.pin_pclk     = PCLK_GPIO_NUM;

config.pin_vsync    = VSYNC_GPIO_NUM;

config.pin_href     = HREF_GPIO_NUM;

config.pin_sccb_sda = SIOD_GPIO_NUM;

config.pin_sccb_scl = SIOC_GPIO_NUM;

config.pin_pwdn     = PWDN_GPIO_NUM;

config.pin_reset    = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_RGB565;

config.frame_size   = FRAMESIZE_QQVGA;

config.fb_count     = 1;

config.fb_location  = CAMERA_FB_IN_PSRAM;

config.grab_mode    = CAMERA_GRAB_LATEST;

if (esp_camera_init(&config) != ESP_OK) {

while (1) { Serial.println("Camera Init Failed"); delay(1000); }

}

sensor_t *s = esp_camera_sensor_get();

s->set_whitebal(s, 1);

s->set_saturation(s, 2);

s->set_brightness(s, 0);

s->set_contrast(s, 1);

s->set_vflip(s, 0);

s->set_hmirror(s, 0);

Serial.println("Warming up sensor...");

delay(WARMUP_MS);

s->set_whitebal(s, 0);

t0 = millis();

Serial.println("time_ms,det_count,colour_id,x,y,w,h");

}

void loop() {

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) return;

uint32_t time_ms = millis() - t0;

const int W = fb->width;

const int H = fb->height;

const uint16_t *buf = (const uint16_t*)fb->buf;

int xmin = W, ymin = H, xmax = -1, ymax = -1;

int hits = 0;

const int stride = 2;

const int MIN_HITS = 20;

for (int y = 0; y < H; y += stride) {

int row = y * W;

for (int x = 0; x < W; x += stride) {

uint16_t p = px16(buf, row + x);

if (isTarget(p)) {

hits++;

if (x < xmin) xmin = x;

if (y < ymin) ymin = y;

if (x > xmax) xmax = x;

if (y > ymax) ymax = y;

}

}

}

int det_count = 0;

int cx = -1, cy = -1, bw = -1, bh = -1;

if (hits >= MIN_HITS) {

bw = xmax - xmin;

bh = ymax - ymin;

if (bw >= 15 && bh >= 15) {

det_count = 1;

cx = xmin + bw / 2;

cy = ymin + bh / 2;

} else {

bw = bh = cx = cy = -1;

}

}

Serial.printf("%lu,%d,%d,%d,%d,%d,%d\n",

time_ms, det_count, TARGET_COLOUR, cx, cy, bw, bh);

esp_camera_fb_return(fb);

delay(10);

}

ESP32-S3 Zero Colour Block Detection (Blue) Test Results

Platform: Waveshare ESP32-S3 Zero + OV2640 Camera (160×120 RGB565)

Algorithm: Pixel-wise RGB565 colour segmentation | Empirical thresholds: B > 220, G > 180, R < 80

The Waveshare ESP32-S3 Zero was configured with an OV2640 camera module for blue (cyan-teal) colour block detection. Detection thresholds were derived empirically from live sensor readings of the target card and background surfaces. This approach was necessary because the target’s cyan-teal hue produces near-saturated blue and green channels simultaneously, requiring data-driven threshold derivation rather than a standard pure-blue condition. Three independent runs were conducted under the same fixed camera rig used across all platforms.

Run Conditions (Blue/Cyan-Teal Target)

- Run 1 (Static): Blue card present at fixed distance with minimal movement.

- Run 2 (Negative control): Blue card removed completely from the scene.

- Run 3 (Dynamic): Card moved left/right/up/down for 30 s, then towards and away from the camera for 30 s.

Table 1. Per-Run Benchmark Summary — Colour Block Detection (Blue)

| Run | Condition | Samples (N) | Duration (s) | FPS | Valid Detections | Detection Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Static | 774 | 69.455 | 11.144 | 774 | 100.000 |
| 2 | Negative Control | 818 | 65.334 | 12.520 | 0 | 0.000 (FPR) |
| 3 | Dynamic | 749 | 66.800 | 11.213 | 749 | 100.000 |
| Combined | All Runs | 2,341 | 201.589 | 11.614 | 1,523 | — |

Table 2. Bounding Box Statistics (Valid Detections Only) — Blue

| Metric (pixels) | Run 1 — Static (Mean ± SD, Min–Max) | Run 3 — Dynamic (Mean ± SD, Min–Max) | Overall (Mean ± SD) |
| --- | --- | --- | --- |
| x (centre) | 89.652 ± 6.441 (69–98) | 79.615 ± 18.796 (44–133) | 84.716 ± 14.828 |
| y (centre) | 43.488 ± 3.648 (32–53) | 39.183 ± 10.809 (6–63) | 41.371 ± 8.296 |
| w (width) | 111.920 ± 10.701 (80–124) | 106.029 ± 23.719 (20–156) | 109.023 ± 18.529 |
| h (height) | 83.385 ± 6.886 (62–102) | 71.963 ± 19.226 (12–116) | 77.768 ± 15.439 |

Combined Summary

- Total samples: 2,341 over 201.589 s. Weighted average FPS: 11.614 FPS.

- Run 1 (Static): 100.000% detection rate — all 774 frames produced a valid bounding box.

- Run 2 (Negative control): 0.000% false positive rate — 0 spurious detections across 818 frames. ✔

- Run 3 (Dynamic): 100.000% detection rate across 749 frames — the strongest dynamic result of all three colours tested.

- Blue achieved the most consistent results across all three colours, with 100% detection in both static and dynamic conditions and 0% false positives, attributable to the high spectral separability of the cyan-teal target (B≈247–255, G≈239) from all background surfaces.

Discussion

The blue (cyan-teal) detection results were the strongest of the three colour targets tested. The final detection thresholds (B > 220, G > 180, R < 80) were derived empirically using a dedicated RGB debug sketch that sampled the centre pixel across four surface types: the cyan-teal target, a red card, a green card, and skin. This data-driven calibration approach revealed that the OV2640 renders the cyan-teal hue with near-saturated blue and green channels simultaneously (B≈247–255, G≈239, R≈41–57), while all other surfaces fall well below B = 220. This single-condition separation eliminated false positives entirely. The iterative threshold development process highlights the importance of per-sensor, per-target threshold calibration in embedded colour detection systems, consistent with findings in the embedded vision literature regarding the sensitivity of RGB-based detectors to sensor-specific colour response curves [3].

Setup Difficulties and Adjustments

- Multiple detection threshold iterations were required before a working blue detector was achieved. Initial thresholds based on standard blue/cyan RGB heuristics produced false positives on red and green surfaces due to the target’s dual-channel (B+G) spectral signature.

- A dedicated RGB debug sketch was developed and deployed to capture live per-pixel R, G, B values from all four test surfaces. The empirically derived thresholds from this calibration step eliminated all false positives.

Conclusion

The ESP32-S3 Zero achieved a weighted average of 11.614 FPS across three blue colour detection benchmark runs, with 100% detection rates in both static and dynamic conditions and 0% false positive rate. These results represent the strongest colour detection performance of the three colours tested, achieved through empirical threshold calibration using live sensor data.

#include "esp_camera.h"

#define RUN_NUMBER      1

#define TARGET_COLOUR   3

#define RGB565_SWAP     1

#define WARMUP_MS       5000

#define PWDN_GPIO_NUM    16

#define RESET_GPIO_NUM   15

#define XCLK_GPIO_NUM     6

#define SIOD_GPIO_NUM     1

#define SIOC_GPIO_NUM     2

#define Y9_GPIO_NUM      14

#define Y8_GPIO_NUM      13

#define Y7_GPIO_NUM      12

#define Y6_GPIO_NUM      11

#define Y5_GPIO_NUM      10

#define Y4_GPIO_NUM       9

#define Y3_GPIO_NUM       8

#define Y2_GPIO_NUM       7

#define VSYNC_GPIO_NUM    3

#define HREF_GPIO_NUM     4

#define PCLK_GPIO_NUM     5

static uint32_t t0 = 0;

static inline uint16_t px16(const uint16_t *buf, int idx) {

uint16_t p = buf[idx];

#if RGB565_SWAP

p = (p >> 8) | (p << 8);

#endif

return p;

}

static inline bool isTarget(uint16_t p) {

uint8_t R  = (p >> 11) & 0x1F; R  = (R  << 3) | (R  >> 2);

uint8_t G  = (p >>  5) & 0x3F; G  = (G  << 2) | (G  >> 4);

uint8_t B  = (p      ) & 0x1F; B  = (B  << 3) | (B  >> 2);

// From real sensor data:

// Cyan:  R=41-57,  G=239,     B=247-255  → B>220 AND G>180 AND R<80

// Green: R=0-49,  G=195-239, B=148-206  → fails B>220

// Red:   R=66-115,G=32-77,   B=41-82    → fails G>180 and B>220

// Skin:  R=82-107,G=97-134,  B=99-132   → fails G>180 and B>220

return (B > 220) && (G > 180) && (R < 80);

}

void setup() {

Serial.begin(115200);

delay(500);

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer   = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM; config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM; config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM; config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM; config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk     = XCLK_GPIO_NUM;

config.pin_pclk     = PCLK_GPIO_NUM;

config.pin_vsync    = VSYNC_GPIO_NUM;

config.pin_href     = HREF_GPIO_NUM;

config.pin_sccb_sda = SIOD_GPIO_NUM;

config.pin_sccb_scl = SIOC_GPIO_NUM;

config.pin_pwdn     = PWDN_GPIO_NUM;

config.pin_reset    = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_RGB565;

config.frame_size   = FRAMESIZE_QQVGA;

config.fb_count     = 1;

config.fb_location  = CAMERA_FB_IN_PSRAM;

config.grab_mode    = CAMERA_GRAB_LATEST;

if (esp_camera_init(&config) != ESP_OK) {

while (1) { Serial.println("Camera Init Failed"); delay(1000); }

}

sensor_t *s = esp_camera_sensor_get();

s->set_whitebal(s, 1);

s->set_saturation(s, 2);

s->set_brightness(s, 0);

s->set_contrast(s, 1);

s->set_vflip(s, 0);

s->set_hmirror(s, 0);

Serial.println("Warming up...");

delay(WARMUP_MS);

s->set_whitebal(s, 0);

t0 = millis();

Serial.println("time_ms,det_count,colour_id,x,y,w,h");

}

void loop() {

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) return;

uint32_t time_ms = millis() - t0;

const int W = fb->width;

const int H = fb->height;

const uint16_t *buf = (const uint16_t*)fb->buf;

int xmin = W, ymin = H, xmax = -1, ymax = -1;

int hits = 0;

for (int y = 0; y < H; y += 2) {

int row = y * W;

for (int x = 0; x < W; x += 2) {

if (isTarget(px16(buf, row + x))) {

hits++;

if (x < xmin) xmin = x;

if (y < ymin) ymin = y;

if (x > xmax) xmax = x;

if (y > ymax) ymax = y;

}

}

}

int det_count = 0;

int colour_id = -1;

int cx = -1, cy = -1, bw = -1, bh = -1;

if (hits > 30 && xmax >= 0) {

int tmp_w = xmax - xmin;

int tmp_h = ymax - ymin;

if (tmp_w > 10 && tmp_h > 10) {

det_count = 1;

colour_id = TARGET_COLOUR;

bw = tmp_w;

bh = tmp_h;

cx = xmin + (bw / 2);

cy = ymin + (bh / 2);

}

}

Serial.print(millis() - t0); Serial.print(",");

Serial.print(det_count);     Serial.print(",");

Serial.print(colour_id);     Serial.print(",");

Serial.print(cx);            Serial.print(",");

Serial.print(cy);            Serial.print(",");

Serial.print(bw);            Serial.print(",");

Serial.println(bh);

esp_camera_fb_return(fb);

delay(20);

}

ESP32-S3 Zero Object Tracking Test Results

Platform: Waveshare ESP32-S3 Zero + OV2640 Camera (160×120 RGB565)

Algorithm: Pixel-wise RGB565 centroid tracking | Target: Vaseline Lip Therapy tin (blue lid)

The Waveshare ESP32-S3 Zero was configured with an OV2640 camera module operating at 160×120 resolution. Object tracking was implemented via pixel-wise RGB565 centroid tracking, consistent with the approach applied on the XIAO ESP32-S3 Sense platform. The bounding box centroid coordinates (cx, cy) reported per frame constitute the tracking output. The target object was the blue lid of a Vaseline Lip Therapy tin, selected for its distinct teal-blue colour. Detection thresholds were derived empirically: R < 20, G > 125, B > 125, with G and B within 30 units of each other. Camera gain and white balance were locked after the warmup period to maintain consistent colour response across all runs.

Run Conditions (Blue Lid Target)

- Run 1 (Static): Blue lid present at fixed position with minimal movement.

- Run 2 (Negative control): Blue lid removed completely from the scene.

- Run 3 (Dynamic): Blue lid moved left/right/up/down for 30 s, then towards and away from the camera for 30 s.

Table 1. Per-Run Benchmark Summary — Object Tracking

| Run | Condition | Samples (N) | Duration (s) | FPS | Valid Detections | Detection Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Static | 784 | 64.150 | 12.221 | 782 | 99.745 |
| 2 | Negative Control | 699 | 64.652 | 10.812 | 29 | 4.149 (FPR) |
| 3 | Dynamic | 765 | 67.457 | 11.341 | 468 | 61.176 |
| Combined | All Runs | 2,248 | 196.259 | 11.454 | 1,279 | — |

Table 2. Bounding Box Statistics (Valid Detections Only) — Object Tracking

| Metric (pixels) | Run 1 — Static (Mean ± SD, Min–Max) | Run 3 — Dynamic (Mean ± SD, Min–Max) | Overall (Mean ± SD) |
| --- | --- | --- | --- |
| x (centre) | 62.881 ± 6.476 (49–87) | 58.333 ± 9.116 (45–112) | 61.317 ± 7.846 |
| y (centre) | 66.827 ± 5.516 (58–77) | 58.607 ± 9.117 (36–95) | 63.826 ± 8.047 |
| w (width) | 94.643 ± 13.607 (54–136) | 103.927 ± 14.393 (61–143) | 98.956 ± 15.454 |
| h (height) | 59.257 ± 11.291 (37–81) | 89.013 ± 16.535 (32–107) | 71.140 ± 20.117 |

Combined Summary

- Total samples: 2,248 over 196.259 s. Weighted average FPS: 11.454 FPS.

- Run 1 (Static): 99.745% detection rate — 782 of 784 frames produced a valid bounding box centroid.

- Run 2 (Negative control): 4.149% false positive rate — 29 of 699 frames triggered spurious detections. Bounding box widths of 134–136 pixels (84–85% of frame width) indicate scattered background noise rather than compact object detections. Documented as a platform finding.

- Run 3 (Dynamic): 61.176% detection rate across 765 frames. Tracking losses attributable to the lid occupying fewer qualifying pixels than the hit threshold at extended distances.

Discussion

The ESP32-S3 Zero achieved approximately 11.454 FPS weighted average across all tracking runs. The centroid-based tracking approach — reporting (cx, cy) per frame — is consistent with the lightweight object tracking methodology applied on the XIAO ESP32-S3 Sense, enabling direct cross-platform comparison.

The 4.149% false positive rate in Run 2 is attributable to ambient lighting causing the black desk mat to produce scattered teal-tinted pixel values passing the colour threshold. The resulting bounding boxes w≈135 px spanning the majority of the 160×120 frame are characteristic of scattered noise rather than compact object detections. The 61.176% dynamic detection rate reflects the fundamental constraint of pixel-wise centroid tracking at 160×120 resolution: at sufficient distance the lid subtends fewer pixels than the hit threshold, causing detection loss — consistent with XIAO ESP32-S3 Sense tracking behaviour [3].

Setup Difficulties and Adjustments

- Multiple threshold iterations were required to balance detection sensitivity at distance against false positives on the black desk mat under variable overhead lighting. A dedicated RGB debug sketch was used to capture live per-pixel values from the target and background surfaces.

- Camera gain and white balance were locked after warmup using set_gain_ctrl(s, 0) and set_whitebal(s, 0) to prevent auto-exposure shifts from altering pixel values mid-session.

Conclusion

The ESP32-S3 Zero achieved a weighted average of 11.454 FPS across three object tracking benchmark runs, with a 99.745% static detection rate and 4.149% false positive rate. The false positive frames were identifiable by their large bounding box dimensions, consistent with background noise. Dynamic tracking achieved 61.176%, confirming the platform’s capability for real-time centroid-based object tracking within the constraints of its embedded architecture.

#include "esp_camera.h"

#define RUN_NUMBER      1

#define TARGET_COLOUR   3

#define RGB565_SWAP     1

#define WARMUP_MS       5000

#define PWDN_GPIO_NUM    16

#define RESET_GPIO_NUM   15

#define XCLK_GPIO_NUM     6

#define SIOD_GPIO_NUM     1

#define SIOC_GPIO_NUM     2

#define Y9_GPIO_NUM      14

#define Y8_GPIO_NUM      13

#define Y7_GPIO_NUM      12

#define Y6_GPIO_NUM      11

#define Y5_GPIO_NUM      10

#define Y4_GPIO_NUM       9

#define Y3_GPIO_NUM       8

#define Y2_GPIO_NUM       7

#define VSYNC_GPIO_NUM    3

#define HREF_GPIO_NUM     4

#define PCLK_GPIO_NUM     5

static uint32_t t0 = 0;

static inline uint16_t px16(const uint16_t *buf, int idx) {

uint16_t p = buf[idx];

#if RGB565_SWAP

p = (p >> 8) | (p << 8);

#endif

return p;

}

static inline bool isTarget(uint16_t p) {

uint8_t R = (p >> 11) & 0x1F; R = (R << 3) | (R >> 2);

uint8_t G = (p >>  5) & 0x3F; G = (G << 2) | (G >> 4);

uint8_t B = (p      ) & 0x1F; B = (B << 3) | (B >> 2);

return (R < 20) && (G > 125) && (B > 125) && (G < B + 30) && (B < G + 30);

}

void setup() {

Serial.begin(115200);

delay(500);

camera_config_t config;

config.ledc_channel = LEDC_CHANNEL_0;

config.ledc_timer   = LEDC_TIMER_0;

config.pin_d0 = Y2_GPIO_NUM; config.pin_d1 = Y3_GPIO_NUM;

config.pin_d2 = Y4_GPIO_NUM; config.pin_d3 = Y5_GPIO_NUM;

config.pin_d4 = Y6_GPIO_NUM; config.pin_d5 = Y7_GPIO_NUM;

config.pin_d6 = Y8_GPIO_NUM; config.pin_d7 = Y9_GPIO_NUM;

config.pin_xclk     = XCLK_GPIO_NUM;

config.pin_pclk     = PCLK_GPIO_NUM;

config.pin_vsync    = VSYNC_GPIO_NUM;

config.pin_href     = HREF_GPIO_NUM;

config.pin_sccb_sda = SIOD_GPIO_NUM;

config.pin_sccb_scl = SIOC_GPIO_NUM;

config.pin_pwdn     = PWDN_GPIO_NUM;

config.pin_reset    = RESET_GPIO_NUM;

config.xclk_freq_hz = 20000000;

config.pixel_format = PIXFORMAT_RGB565;

config.frame_size   = FRAMESIZE_QQVGA;

config.fb_count     = 1;

config.fb_location  = CAMERA_FB_IN_PSRAM;

config.grab_mode    = CAMERA_GRAB_LATEST;

if (esp_camera_init(&config) != ESP_OK) {

while (1) { Serial.println("Camera Init Failed"); delay(1000); }

}

sensor_t *s = esp_camera_sensor_get();

s->set_whitebal(s, 1);

s->set_saturation(s, 2);

s->set_brightness(s, 1);

s->set_contrast(s, 1);

s->set_vflip(s, 0);

s->set_hmirror(s, 0);

Serial.println("Warming up...");

delay(WARMUP_MS);

s->set_whitebal(s, 0);

s->set_gain_ctrl(s, 0);

t0 = millis();

Serial.println("time_ms,det_count,colour_id,x,y,w,h");

}

void loop() {

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) return;

uint32_t time_ms = millis() - t0;

const int W = fb->width;

const int H = fb->height;

const uint16_t *buf = (const uint16_t*)fb->buf;

int xmin = W, ymin = H, xmax = -1, ymax = -1;

int hits = 0;

for (int y = 0; y < H; y++) {

int row = y * W;

for (int x = 0; x < W; x++) {

if (isTarget(px16(buf, row + x))) {

hits++;

if (x < xmin) xmin = x;

if (y < ymin) ymin = y;

if (x > xmax) xmax = x;

if (y > ymax) ymax = y;

}

}

}

int det_count = 0;

int colour_id = -1;

int cx = -1, cy = -1, bw = -1, bh = -1;

if (hits > 30 && xmax >= 0) {

int tmp_w = xmax - xmin;

int tmp_h = ymax - ymin;

// Background under bright light spans nearly full frame — reject it

// Lid is always smaller than 90% of frame in both dimensions

bool valid_size = (tmp_w > 5) && (tmp_h > 5) &&

(tmp_w < W * 0.90) && (tmp_h < H * 0.90);

if (valid_size) {

det_count = 1;

colour_id = TARGET_COLOUR;

bw = tmp_w;

bh = tmp_h;

cx = xmin + (bw / 2);

cy = ymin + (bh / 2);

}

}

Serial.print(millis() - t0); Serial.print(",");

Serial.print(det_count);     Serial.print(",");

Serial.print(colour_id);     Serial.print(",");

Serial.print(cx);            Serial.print(",");

Serial.print(cy);            Serial.print(",");

Serial.print(bw);            Serial.print(",");

Serial.println(bh);

esp_camera_fb_return(fb);

delay(20);

}

ESP32-S3 Zero Face Detection Test Results

Platform: Waveshare ESP32-S3 Zero + OV2640 Camera (160×120 → 96×96 RGB888)

Model: Edge Impulse FOMO (MobileNetV2 0.1×) | Inference resolution: 96×96

The Waveshare ESP32-S3 Zero was configured with an OV2640 camera module operating at 160×120 resolution. Face detection was performed using an Edge Impulse FOMO model based on MobileNetV2 0.1×, trained to detect human faces [3]. The same model was deployed on the XIAO ESP32-S3 Sense platform, enabling direct cross-platform inference comparison. Frames were captured in RGB565 format, converted to RGB888 via manual bit-shifting, and resized from 160×120 to the model’s 96×96 input resolution using Edge Impulse’s crop_and_interpolate_rgb888 function. Three independent runs were conducted under the fixed camera rig methodology applied across all platforms.

Run Conditions

- Run 1 (Static): Subject’s face present at fixed distance with minimal movement.

- Run 2 (Negative control): No face present in the scene — camera pointed at empty background.

- Run 3 (Dynamic): Face moved left/right/up/down for 30 s, then towards and away from the camera for 30 s.

Table 1. Per-Run Benchmark Summary — Face Detection

| Run | Condition | Samples (N) | Duration (s) | FPS | Valid Detections | Detection Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Static | 297 | 68.797 | 4.317 | 251 | 84.512 |
| 2 | Negative Control | 291 | 109.760 | 2.651 | 0 | 0.000 (FPR) |
| 3 | Dynamic | 290 | 67.145 | 4.319 | 170 | 58.621 |
| Combined | All Runs | 878 | 245.702 | 3.573 | 421 | — |

Table 2. Bounding Box Statistics (Valid Detections Only) — Face Detection

| Metric (pixels) | Run 1 — Static (Mean ± SD, Min–Max) | Run 3 — Dynamic (Mean ± SD, Min–Max) | Overall (Mean ± SD) |
| --- | --- | --- | --- |
| x (centre) | 44.000 ± 0.000 (44–44) | 48.094 ± 15.061 (12–84) | 45.653 ± 9.763 |
| y (centre) | 12.000 ± 0.000 (12–12) | 18.659 ± 8.426 (12–44) | 14.689 ± 6.266 |
| w (width) | 8.000 ± 0.000 (8–8) | 8.094 ± 0.865 (8–16) | 8.038 ± 0.551 |
| h (height) | 8.000 ± 0.000 (8–8) | 9.365 ± 3.018 (8–16) | 8.551 ± 2.028 |

Combined Summary

- Total samples: 878 over 245.702 s. Weighted average FPS: 3.573 FPS.

- Run 1 (Static): 84.512% detection rate — 251 of 297 frames produced a valid face detection bounding box.

- Run 2 (Negative control): 0.000% false positive rate. The reduced FPS of 2.651 is attributable to the inference loop completing without bounding box output rows, resulting in fewer timestamped CSV entries per unit time. ✔

- Run 3 (Dynamic): 58.621% detection rate. Detection losses occurred during fast lateral movement and at extended distances.

- Run 1 bounding boxes fixed at x=44, y=12, w=8, h=8 across all 251 detections — characteristic of the FOMO grid output architecture, identical to the XIAO ESP32-S3 Sense behaviour.

Discussion

The ESP32-S3 Zero achieved approximately 4.3 FPS in active detection runs, closely matching the XIAO ESP32-S3 Sense (~4.4 FPS). This similarity is expected given both platforms share the same ESP32-S3 processor core running the same FOMO model at identical inference resolution (96×96). The marginal difference is attributable to the lower OV2640 capture resolution (160×120 vs 320×240 on the XIAO) and associated conversion overhead.

The fixed bounding box coordinates in Run 1 reflect the FOMO architecture’s grid-based output: FOMO divides the 96×96 input into an 8×8 grid of 12×12-pixel cells and reports the grid cell containing the detected object centroid, resulting in quantised bounding box positions at multiples of 8 pixels and a fixed minimum box size of 8×8 pixels. This behaviour is consistent with published FOMO architecture documentation and observations on the XIAO ESP32-S3 Sense platform [3].

Setup Difficulties and Adjustments

- The Edge Impulse example sketch uses JPEG capture with fmt2rgb888 conversion. This failed on the ESP32-S3 Zero at QQVGA resolution. The pipeline was modified to capture RGB565 directly and convert to RGB888 via manual per-pixel bit-shifting, which resolved the issue.

- The camera frame size was set to QQVGA (160×120) rather than QVGA (320×240) to fit within PSRAM constraints on the Zero.

- The Huge APP partition scheme was required in Arduino IDE to accommodate the combined footprint of the esp_camera library and the Edge Impulse FOMO inference library.

Conclusion

The ESP32-S3 Zero achieved a weighted average of 3.573 FPS across three face detection benchmark runs, with an 84.512% static detection rate, 0.000% false positive rate, and 58.621% dynamic detection rate. Inference speed of approximately 4.3 FPS closely matches the XIAO ESP32-S3 Sense, confirming that FOMO inference performance is primarily determined by the processor core rather than the camera module.

#include <Face_detection_-_FOMO_inferencing.h>

#include "edge-impulse-sdk/dsp/image/image.hpp"

#include "esp_camera.h"

#define PWDN_GPIO_NUM    16

#define RESET_GPIO_NUM   15

#define XCLK_GPIO_NUM     6

#define SIOD_GPIO_NUM     1

#define SIOC_GPIO_NUM     2

#define Y9_GPIO_NUM      14

#define Y8_GPIO_NUM      13

#define Y7_GPIO_NUM      12

#define Y6_GPIO_NUM      11

#define Y5_GPIO_NUM      10

#define Y4_GPIO_NUM       9

#define Y3_GPIO_NUM       8

#define Y2_GPIO_NUM       7

#define VSYNC_GPIO_NUM    3

#define HREF_GPIO_NUM     4

#define PCLK_GPIO_NUM     5

#define EI_CAMERA_RAW_FRAME_BUFFER_COLS  160

#define EI_CAMERA_RAW_FRAME_BUFFER_ROWS  120

#define EI_CAMERA_FRAME_BYTE_SIZE         3

#define RUN_NUMBER    1

#define WARMUP_MS     5000

static bool is_initialised = false;

static uint8_t *snapshot_buf = nullptr;

static uint32_t t0 = 0;

static camera_config_t camera_config = {

.pin_pwdn     = PWDN_GPIO_NUM,

.pin_reset    = RESET_GPIO_NUM,

.pin_xclk     = XCLK_GPIO_NUM,

.pin_sccb_sda = SIOD_GPIO_NUM,

.pin_sccb_scl = SIOC_GPIO_NUM,

.pin_d7       = Y9_GPIO_NUM,

.pin_d6       = Y8_GPIO_NUM,

.pin_d5       = Y7_GPIO_NUM,

.pin_d4       = Y6_GPIO_NUM,

.pin_d3       = Y5_GPIO_NUM,

.pin_d2       = Y4_GPIO_NUM,

.pin_d1       = Y3_GPIO_NUM,

.pin_d0       = Y2_GPIO_NUM,

.pin_vsync    = VSYNC_GPIO_NUM,

.pin_href     = HREF_GPIO_NUM,

.pin_pclk     = PCLK_GPIO_NUM,

.xclk_freq_hz = 20000000,

.ledc_timer   = LEDC_TIMER_0,

.ledc_channel = LEDC_CHANNEL_0,

.pixel_format = PIXFORMAT_RGB565,   // direct RGB565, no JPEG

.frame_size   = FRAMESIZE_QQVGA,    // 160x120

.fb_count     = 1,

.fb_location  = CAMERA_FB_IN_PSRAM,

.grab_mode    = CAMERA_GRAB_WHEN_EMPTY,

};

bool ei_camera_init(void);

bool ei_camera_capture(uint32_t img_width, uint32_t img_height, uint8_t *out_buf);

static int ei_camera_get_data(size_t offset, size_t length, float *out_ptr);

void setup() {

Serial.begin(115200);

delay(500);

if (!ei_camera_init()) {

Serial.println("Camera init FAILED");

while (1) delay(1000);

}

sensor_t *s = esp_camera_sensor_get();

s->set_whitebal(s, 1);

s->set_saturation(s, 0);

s->set_brightness(s, 1);

s->set_vflip(s, 0);

s->set_hmirror(s, 0);

Serial.println("Warming up sensor...");

delay(WARMUP_MS);

s->set_whitebal(s, 0);

t0 = millis();

Serial.println("time_ms,det_count,class_id,x,y,w,h");

}

void loop() {

snapshot_buf = (uint8_t*)malloc(EI_CAMERA_RAW_FRAME_BUFFER_COLS *

EI_CAMERA_RAW_FRAME_BUFFER_ROWS *

EI_CAMERA_FRAME_BYTE_SIZE);

if (!snapshot_buf) {

Serial.println("ERR: malloc failed");

return;

}

ei::signal_t signal;

signal.total_length = EI_CLASSIFIER_INPUT_WIDTH * EI_CLASSIFIER_INPUT_HEIGHT;

signal.get_data     = &ei_camera_get_data;

if (!ei_camera_capture(EI_CLASSIFIER_INPUT_WIDTH, EI_CLASSIFIER_INPUT_HEIGHT, snapshot_buf)) {

Serial.println("ERR: capture failed");

free(snapshot_buf);

return;

}

ei_impulse_result_t result = { 0 };

EI_IMPULSE_ERROR err = run_classifier(&signal, &result, false);

if (err != EI_IMPULSE_OK) {

Serial.print("ERR: classifier failed: ");

Serial.println(err);

free(snapshot_buf);

return;

}

uint32_t time_ms = millis() - t0;

int det_count = 0;

for (uint32_t i = 0; i < result.bounding_boxes_count; i++) {

if (result.bounding_boxes[i].value > 0) det_count++;

}

if (det_count == 0) {

Serial.print(time_ms); Serial.print(",0,-1,-1,-1,-1,-1");

Serial.println();

} else {

for (uint32_t i = 0; i < result.bounding_boxes_count; i++) {

ei_impulse_result_bounding_box_t bb = result.bounding_boxes[i];

if (bb.value == 0) continue;

Serial.print(time_ms);   Serial.print(",");

Serial.print(det_count); Serial.print(",");

Serial.print(1);         Serial.print(",");

Serial.print(bb.x + bb.width  / 2); Serial.print(",");

Serial.print(bb.y + bb.height / 2); Serial.print(",");

Serial.print(bb.width);  Serial.print(",");

Serial.println(bb.height);

}

}

free(snapshot_buf);

}

bool ei_camera_init(void) {

if (is_initialised) return true;

esp_err_t err = esp_camera_init(&camera_config);

if (err != ESP_OK) {

Serial.printf("Camera init failed: 0x%x\n", err);

return false;

}

is_initialised = true;

return true;

}

bool ei_camera_capture(uint32_t img_width, uint32_t img_height, uint8_t *out_buf) {

if (!is_initialised) return false;

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) return false;

// Convert RGB565 → RGB888 manually

uint16_t *src = (uint16_t*)fb->buf;

uint8_t  *dst = out_buf;

for (int i = 0; i < EI_CAMERA_RAW_FRAME_BUFFER_COLS * EI_CAMERA_RAW_FRAME_BUFFER_ROWS; i++) {

uint16_t px = src[i];

// swap bytes for OV2640

px = (px >> 8) | (px << 8);

dst[i*3 + 0] = ((px >> 11) & 0x1F) << 3; // R

dst[i*3 + 1] = ((px >>  5) & 0x3F) << 2; // G

dst[i*3 + 2] = ( px        & 0x1F) << 3; // B

}

esp_camera_fb_return(fb);

// Crop and resize to EI input size (96x96)

if (img_width != EI_CAMERA_RAW_FRAME_BUFFER_COLS ||

img_height != EI_CAMERA_RAW_FRAME_BUFFER_ROWS) {

ei::image::processing::crop_and_interpolate_rgb888(

out_buf,

EI_CAMERA_RAW_FRAME_BUFFER_COLS,

EI_CAMERA_RAW_FRAME_BUFFER_ROWS,

out_buf,

img_width,

img_height);

}

return true;

}

static int ei_camera_get_data(size_t offset, size_t length, float *out_ptr) {

size_t pixel_ix   = offset * 3;

size_t out_ptr_ix = 0;

while (out_ptr_ix < length) {

out_ptr[out_ptr_ix] = (snapshot_buf[pixel_ix + 2] << 16) +

(snapshot_buf[pixel_ix + 1] <<  8) +

snapshot_buf[pixel_ix];

out_ptr_ix++;

pixel_ix += 3;

}

return 0;

}

#if !defined(EI_CLASSIFIER_SENSOR) || EI_CLASSIFIER_SENSOR != EI_CLASSIFIER_SENSOR_CAMERA

#error "Invalid model for current sensor"

#endif

ESP32-S3 Zero Object Classification Test Results

Platform: Waveshare ESP32-S3 Zero + OV2640 Camera (160×120 → 96×96 RGB888)

Model: Edge Impulse FOMO MobileNetV2 0.35 | Classes: OBJECTA (Vaseline tin), OBJECTB (Huawei case), OBJECTC (BIC pens)

The Waveshare ESP32-S3 Zero was configured with an OV2640 camera module operating at 160×120 resolution. Object classification was performed using the same Edge Impulse FOMO model (MobileNetV2 0.35, 88.6% F1 score on validation set) deployed on the XIAO ESP32-S3 Sense platform [3]. The model was originally trained on smartphone camera images labelled with three object classes: OBJECTA (Vaseline Lip Therapy tin), OBJECTB (Huawei FreeBuds earbuds case), and OBJECTC (BIC pen pack), plus a BACKGROUND class. Frames were captured in RGB565 format, converted to RGB888 via manual bit-shifting, and resized from 160×120 to the model’s 96×96 input resolution. Three independent runs were conducted under the fixed camera rig methodology applied across all platforms.

Run Conditions

- Run 1 (Static): Objects A, B, and C presented sequentially for approximately 25 seconds each at fixed distance.

- Run 2 (Negative control): No target object present — camera pointed at empty background.

- Run 3 (Dynamic): Objects swapped and moved across the frame at varying distances and orientations.

Table 1. Per-Run Benchmark Summary — Object Classification

| Run | Condition | Samples (N) | Duration (s) | FPS | Valid Detections | Detection Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Static (A→B→C) | 376 | 86.097 | 4.367 | 101 | 26.862 |
| 2 | Negative Control | 280 | 64.949 | 4.311 | 0 | 0.000 (FPR) |
| 3 | Dynamic | 405 | 93.508 | 4.331 | 41 | 10.123 |
| Combined | All Runs | 1,061 | 244.554 | 4.339 | 142 | — |

Table 2. Class Detection Breakdown (Valid Detections Only)

| Run | OBJECTA — Vaseline tin | OBJECTB — Huawei case | OBJECTC — BIC pens | Total Detections |
| --- | --- | --- | --- | --- |
| Run 1 (Static) | 45 (44.6%) | 30 (29.7%) | 26 (25.7%) | 101 |
| Run 2 (Negative) | 0 | 0 | 0 | 0 |
| Run 3 (Dynamic) | 0 (0%) | 26 (63.4%) | 15 (36.6%) | 41 |
| Overall | 45 (31.7%) | 56 (39.4%) | 41 (28.9%) | 142 |

Combined Summary

- Total samples: 1,061 over 244.554 s. Weighted average FPS: 4.339 FPS.

- Run 1 (Static): 26.862% overall detection rate. All three classes detected: OBJECTA=45, OBJECTB=30, OBJECTC=26.

- Run 2 (Negative control): 0.000% false positive rate — no spurious detections across 280 frames. ✔

- Run 3 (Dynamic): 10.123% detection rate. OBJECTA was not detected in Run 3; OBJECTB and OBJECTC detected with class confusion present.

- Combined false positive rate: 0.000% — the model did not trigger on empty background across any run.

Discussion

The ESP32-S3 Zero achieved approximately 4.3 FPS, consistent with the face detection results on the same platform. The 0% false positive rate demonstrates that the model reliably suppresses background detections.

The reduced detection rates (26.9% static, 10.1% dynamic) compared to the XIAO ESP32-S3 Sense (~57% and ~47%) are primarily attributed to domain shift between training and inference data distributions. The FOMO model was trained on smartphone camera images at higher resolution, while inference on the Zero operates on OV2640 frames at 160×120 — a lower resolution with different colour response characteristics. This domain shift is a known challenge in embedded computer vision transfer learning [3].

Class confusion was observed, consistent with the XIAO results. On the Zero, OBJECTA was absent from Run 3 detections entirely, suggesting its learned features are less robust to the resolution and colour differences introduced by the OV2640 at 160×120. The 8×8 pixel FOMO grid output is identical to the XIAO and face detection results, confirming consistent architecture behaviour across tasks.

Setup Difficulties and Adjustments

- The Edge Impulse example sketch’s JPEG capture pipeline failed at QQVGA resolution. The pipeline was adapted to use RGB565 direct capture with manual RGB888 conversion, identical to the face detection adaptation.

- The model was trained on smartphone images rather than images captured directly from the ESP32-S3 Zero OV2640 camera. This training-inference domain mismatch contributed to reduced detection rates. Retraining with images captured directly from the Zero’s camera would be expected to significantly improve detection rates.

- The while(!Serial) blocking call in the Edge Impulse example sketch was removed to allow inference to start automatically without serial monitor connection.

Conclusion

The ESP32-S3 Zero achieved a weighted average of 4.339 FPS across three object classification benchmark runs, with 26.862% static detection rate, 0.000% false positive rate, and 10.123% dynamic detection rate. The reduced detection rates are attributed to training-inference domain shift. The 0% false positive rate and consistent 4.3 FPS inference speed confirm the platform’s capability for FOMO-based object classification within its embedded constraints.

#include <XIAO_Object_Classification_inferencing.h>

#include "edge-impulse-sdk/dsp/image/image.hpp"

#include "esp_camera.h"

#define PWDN_GPIO_NUM    16

#define RESET_GPIO_NUM   15

#define XCLK_GPIO_NUM     6

#define SIOD_GPIO_NUM     1

#define SIOC_GPIO_NUM     2

#define Y9_GPIO_NUM      14

#define Y8_GPIO_NUM      13

#define Y7_GPIO_NUM      12

#define Y6_GPIO_NUM      11

#define Y5_GPIO_NUM      10

#define Y4_GPIO_NUM       9

#define Y3_GPIO_NUM       8

#define Y2_GPIO_NUM       7

#define VSYNC_GPIO_NUM    3

#define HREF_GPIO_NUM     4

#define PCLK_GPIO_NUM     5

#define EI_CAMERA_RAW_FRAME_BUFFER_COLS  160

#define EI_CAMERA_RAW_FRAME_BUFFER_ROWS  120

#define EI_CAMERA_FRAME_BYTE_SIZE         3

#define WARMUP_MS  5000

static bool is_initialised = false;

static uint8_t *snapshot_buf = nullptr;

static uint32_t t0 = 0;

static camera_config_t camera_config = {

.pin_pwdn     = PWDN_GPIO_NUM,

.pin_reset    = RESET_GPIO_NUM,

.pin_xclk     = XCLK_GPIO_NUM,

.pin_sccb_sda = SIOD_GPIO_NUM,

.pin_sccb_scl = SIOC_GPIO_NUM,

.pin_d7       = Y9_GPIO_NUM,

.pin_d6       = Y8_GPIO_NUM,

.pin_d5       = Y7_GPIO_NUM,

.pin_d4       = Y6_GPIO_NUM,

.pin_d3       = Y5_GPIO_NUM,

.pin_d2       = Y4_GPIO_NUM,

.pin_d1       = Y3_GPIO_NUM,

.pin_d0       = Y2_GPIO_NUM,

.pin_vsync    = VSYNC_GPIO_NUM,

.pin_href     = HREF_GPIO_NUM,

.pin_pclk     = PCLK_GPIO_NUM,

.xclk_freq_hz = 20000000,

.ledc_timer   = LEDC_TIMER_0,

.ledc_channel = LEDC_CHANNEL_0,

.pixel_format = PIXFORMAT_RGB565,

.frame_size   = FRAMESIZE_QQVGA,

.fb_count     = 1,

.fb_location  = CAMERA_FB_IN_PSRAM,

.grab_mode    = CAMERA_GRAB_WHEN_EMPTY,

};

bool ei_camera_init(void);

bool ei_camera_capture(uint32_t img_width, uint32_t img_height, uint8_t *out_buf);

static int ei_camera_get_data(size_t offset, size_t length, float *out_ptr);

void setup() {

Serial.begin(115200);

delay(500); // NO while(!Serial) — that freezes everything

if (!ei_camera_init()) {

Serial.println("Camera init FAILED");

while (1) delay(1000);

}

sensor_t *s = esp_camera_sensor_get();

s->set_whitebal(s, 1);

s->set_saturation(s, 0);

s->set_brightness(s, 1);

s->set_vflip(s, 0);

s->set_hmirror(s, 0);

Serial.println("Warming up...");

delay(WARMUP_MS);

s->set_whitebal(s, 0);

t0 = millis();

Serial.println("Camera ready. Starting inference...");

Serial.println("time_ms,det_count,class_id,x,y,w,h");

}

void loop() {

snapshot_buf = (uint8_t*)malloc(EI_CAMERA_RAW_FRAME_BUFFER_COLS *

EI_CAMERA_RAW_FRAME_BUFFER_ROWS *

EI_CAMERA_FRAME_BYTE_SIZE);

if (!snapshot_buf) {

Serial.println("ERR: malloc failed");

return;

}

ei::signal_t signal;

signal.total_length = EI_CLASSIFIER_INPUT_WIDTH * EI_CLASSIFIER_INPUT_HEIGHT;

signal.get_data     = &ei_camera_get_data;

if (!ei_camera_capture(EI_CLASSIFIER_INPUT_WIDTH, EI_CLASSIFIER_INPUT_HEIGHT, snapshot_buf)) {

Serial.println("ERR: capture failed");

free(snapshot_buf);

return;

}

ei_impulse_result_t result = { 0 };

EI_IMPULSE_ERROR err = run_classifier(&signal, &result, false);

if (err != EI_IMPULSE_OK) {

Serial.print("ERR: classifier failed: ");

Serial.println(err);

free(snapshot_buf);

return;

}

uint32_t time_ms = millis() - t0;

#if EI_CLASSIFIER_OBJECT_DETECTION == 1

// FOMO object detection output — same as face detection

int det_count = 0;

for (uint32_t i = 0; i < result.bounding_boxes_count; i++) {

if (result.bounding_boxes[i].value > 0) det_count++;

}

if (det_count == 0) {

Serial.print(time_ms);

Serial.println(",0,-1,-1,-1,-1,-1");

} else {

for (uint32_t i = 0; i < result.bounding_boxes_count; i++) {

ei_impulse_result_bounding_box_t bb = result.bounding_boxes[i];

if (bb.value == 0) continue;

// Map label name to class_id number

int class_id = 1; // default

if      (strcmp(bb.label, "OBJECTA") == 0) class_id = 1;

else if (strcmp(bb.label, "OBJECTB") == 0) class_id = 2;

else if (strcmp(bb.label, "OBJECTC") == 0) class_id = 3;

Serial.print(time_ms);   Serial.print(",");

Serial.print(det_count); Serial.print(",");

Serial.print(class_id);  Serial.print(",");

Serial.print(bb.x + bb.width  / 2); Serial.print(",");

Serial.print(bb.y + bb.height / 2); Serial.print(",");

Serial.print(bb.width);  Serial.print(",");

Serial.println(bb.height);

}

}

#else

// Pure classification output

int best_class = -1;

float best_conf = 0.6; // minimum confidence threshold

for (int i = 0; i < EI_CLASSIFIER_LABEL_COUNT; i++) {

if (result.classification[i].value > best_conf) {

best_conf = result.classification[i].value;

best_class = i;

}

}

int det_count = (best_class > 0) ? 1 : 0;

int class_id  = (best_class > 0) ? best_class : -1;

Serial.print(time_ms);   Serial.print(",");

Serial.print(det_count); Serial.print(",");

Serial.print(class_id);  Serial.print(",");

Serial.println("-1,-1,-1,-1");

#endif

free(snapshot_buf);

}

bool ei_camera_init(void) {

if (is_initialised) return true;

esp_err_t err = esp_camera_init(&camera_config);

if (err != ESP_OK) {

Serial.printf("Camera init failed: 0x%x\n", err);

return false;

}

is_initialised = true;

return true;

}

bool ei_camera_capture(uint32_t img_width, uint32_t img_height, uint8_t *out_buf) {

if (!is_initialised) return false;

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) return false;

// RGB565 → RGB888 manual conversion with byte swap

uint16_t *src = (uint16_t*)fb->buf;

uint8_t  *dst = out_buf;

for (int i = 0; i < EI_CAMERA_RAW_FRAME_BUFFER_COLS * EI_CAMERA_RAW_FRAME_BUFFER_ROWS; i++) {

uint16_t px = src[i];

px = (px >> 8) | (px << 8);

dst[i*3 + 0] = ((px >> 11) & 0x1F) << 3;

dst[i*3 + 1] = ((px >>  5) & 0x3F) << 2;

dst[i*3 + 2] = ( px        & 0x1F) << 3;

}

esp_camera_fb_return(fb);

if (img_width  != EI_CAMERA_RAW_FRAME_BUFFER_COLS ||

img_height != EI_CAMERA_RAW_FRAME_BUFFER_ROWS) {

ei::image::processing::crop_and_interpolate_rgb888(

out_buf,

EI_CAMERA_RAW_FRAME_BUFFER_COLS,

EI_CAMERA_RAW_FRAME_BUFFER_ROWS,

out_buf,

img_width,

img_height);

}

return true;

}

static int ei_camera_get_data(size_t offset, size_t length, float *out_ptr) {

size_t pixel_ix   = offset * 3;

size_t out_ptr_ix = 0;

while (out_ptr_ix < length) {

out_ptr[out_ptr_ix] = (snapshot_buf[pixel_ix + 2] << 16) +

(snapshot_buf[pixel_ix + 1] <<  8) +

snapshot_buf[pixel_ix];

out_ptr_ix++;

pixel_ix += 3;

}

return 0;

}

#if !defined(EI_CLASSIFIER_SENSOR) || EI_CLASSIFIER_SENSOR != EI_CLASSIFIER_SENSOR_CAMERA

#error "Invalid model for current sensor"

#endif

References

The following sources informed the hardware setup and software implementation used across all ESP32-S3 Zero benchmark tasks.

- [1] Waveshare, ‘ESP32-S3-Zero Wiki,’ Waveshare Electronics, 2023. [Link]

Hardware pinout and GPIO mapping reference for the Waveshare ESP32-S3-Zero board. Used to confirm available I2C pins (GP1/GP2), DVP camera pins, and reserved pins (GP21 — WS2812 LED; GP19/20 — USB) during OV2640 wiring and sketch development.

- [2] Espressif Systems, ‘ESP32-S3 Camera Driver (esp_camera),’ GitHub — espressif/esp32-camera, 2023. [Link]

Official camera driver library used to interface the OV2640 with the ESP32-S3’s built-in DVP camera peripheral across all benchmark tasks. The esp_camera_init(), esp_camera_fb_get(), and sensor_t configuration functions are used in all colour detection, tracking, face detection, and object classification sketches.